Dynastart Test

- Remove D+ and DF from the unit

- Connect a good wire to DF that you can earth later

- Connect a voltmeter to D+ and earth

- Disconnect belts and spin up the dynastart to max speed using the start button

- Release button, quickly short DF to earth, note volts and deceleration

- A good dynastart will jump up above 16V, and then decelerate quickly

You can also measure ohms between DF and earth, which should be about 3-6 ohms.